AeroSense

User Guide

Portable Air Quality Monitor

Particulate Matter  •  Combustible Gas  •  Live AQI

Please read this guide before first use.

# 1. Welcome to AeroSense

AeroSense is a compact desktop air quality monitor. It continuously samples the air around it and shows you, at a glance, how clean or polluted that air is right now.

The readings appear on the device itself, and AeroSense is fully usable with no network at all. Cloud sync is optional: a unit with no Wi-Fi details stored never switches its radio on, sends nothing, and shows no connection indicator.

Units that have been given Wi-Fi details also upload their readings to a private cloud database (Google Firebase), so the air quality can be checked from elsewhere and kept as history.

What is sent: the particulate and gas readings, the AQI and its category, the device uptime and Wi-Fi signal strength, and a timestamp. This device has no microphone, no camera and no location sensor, and no personal information is uploaded. The database is private — only the single account the device signs in with can read or write it.

## What AeroSense Measures

| Reading | What it is | Units | Sensor |
| --- | --- | --- | --- |
| PM1.0 | Very fine airborne particles, under 1 micron across. These reach deepest into the lungs. | µg/m³ | PMS5003 |
| PM2.5 | Fine particles under 2.5 microns — smoke, soot, haze, cooking fumes. The single most important health indicator. | µg/m³ | PMS5003 |
| PM10 | Coarser particles under 10 microns — dust, pollen, road grit. | µg/m³ | PMS5003 |
| Gas (MQ-2) | Relative level of combustible gas and smoke — LPG, methane, cooking gas, smoke. | Relative (0–4095) | MQ-2 |
| AQI | Air Quality Index, calculated from PM2.5 and PM10. The headline number on the Home screen. | 0–500 | Calculated |

Note: The gas sensor reports a relative level, not a calibrated concentration. It is excellent at showing change — a rise means something is in the air — but the numbers are not parts-per-million, and they are deliberately excluded from the AQI calculation.

# 2. Quick Start

- Place AeroSense on a flat, dry surface at roughly desk or breathing height, away from direct draughts, radiators and windows.

- Connect the USB power cable. The screen lights up and a start-up screen appears for about 1.5 seconds.

- The Home screen appears, showing a large AQI number and category.

- Allow 60 seconds for the gas sensor to warm up. A high Gas reading during this period is expected — see section 4.

- Use Button 2 to move forward through the pages and Button 1 to move back. Hold Button 1 at any time to return to Home.

That is the whole setup for taking readings. If you want cloud sync as well, put the monitor on your Wi-Fi with Wi-Fi Setup in the Quick Menu — see section 5 — and section 3 describes the indicator that tells you whether it is working.

# 3. Getting to Know the Screen

Every page shares the same three-part layout.

| Area | Location | What it shows |
| --- | --- | --- |
| Header | Top strip | The name of the page you are on, with the sensor and connection indicators at the right-hand end. |
| Body | Middle | The readings for the current page. |
| Footer | Bottom strip | A button reminder — Nav Hold2:Menu — and a row of dots showing which of the seven pages you are on. |

At the right-hand end of the header are two small indicators. The round dot is the particulate sensor: green while it is reporting, red when it is not.

Next to it, on units with cloud sync configured, a four-bar meter shows the Wi-Fi signal strength, and its colour shows whether readings are reaching the database: green for synced, amber for connecting or signing in, red for uploads failing, and grey for no network. A unit without cloud sync shows no meter at all.

# 4. First-Time Startup and Sensor Warm-Up

## What happens when you power on

- The start-up screen appears for about 1.5 seconds.

- The Home screen appears. The display, buttons and particulate sensor are immediately fully usable.

- The particulate sensor (PMS5003) needs a few seconds to send its first reading. Until it does, the AQI panel shows - - - and NO DATA. This normally clears within about 10 seconds.

- The gas sensor begins a 60-second warm-up.

## Why the Gas reading starts very high

The MQ-2 works by heating a small sensing element. Until that heater reaches its working temperature, the sensor reports a reading close to maximum. This is how the hardware behaves — it is not a fault, and it does not mean there is gas in the room.

AeroSense handles this for you:

- The sensor is sampled continuously throughout warm-up, so you can watch the number settle downward.

- Gas warnings are switched off for the full 60 seconds, so you will not get false alarms.

- The warm-up readings are excluded from the overall air quality assessment, so they cannot trigger a false “Poor Air Quality” warning either.

- AQI is unaffected and keeps working normally throughout, because AQI is calculated only from the particulate sensor.

## How to tell warm-up is still running

| Where to look | During warm-up | After warm-up |
| --- | --- | --- |
| Home page — Gas card | warming | The live reading |
| Gas Monitoring — status word | WARMING UP in accent colour | LOW / MEDIUM / HIGH / DANGER |
| Gas Monitoring — bottom line | Warming up... 45s counting down | Line disappears |
| System Status — MQ2 row | Warmup 45s | READY |

Note: Warm-up runs once, from the moment power is applied. It is not restarted by changing pages or pressing buttons. It only runs again if you power the device off and on.

The 60-second period is enough to eliminate the false-alarm spike. For the most stable gas readings, leave AeroSense powered on continuously — this sensor type is designed for permanent operation and becomes steadier the longer it runs.

# 5. Using the Two Buttons

AeroSense has two buttons. Every function is reached with a short press or a press-and-hold.

A short press is a normal quick tap. A hold means keeping the button down for about half a second — the action triggers while you are still holding it.

## Normal use — browsing pages

| Action | Result |
| --- | --- |
| Press Button 1 | Go to the previous page |
| Press Button 2 | Go to the next page |
| Hold Button 1 | Jump straight back to Home |
| Hold Button 2 | Open the Quick Menu |

## Inside the Quick Menu

| Action | Result |
| --- | --- |
| Press Button 1 | Move the highlight up |
| Press Button 2 | Move the highlight down |
| Hold Button 2 | Confirm the highlighted option |
| Hold Button 1 | Cancel and close the menu |

## Quick Menu options

| Option | What it does |
| --- | --- |
| Reset Statistics | Clears the min / average / max figures on the Statistics page and starts them again from zero. |
| Go to Home | Returns to page 1. |
| Wi-Fi Setup | Puts the monitor into Wi-Fi setup, where it raises its own network so a phone can choose which Wi-Fi it should use. |
| Close Menu | Closes the menu without doing anything. |

## Choosing a Wi-Fi network

Wi-Fi Setup in the Quick Menu is how the monitor is put on a network. It has two buttons, which is no way to type a password, so it borrows a phone instead: the monitor raises a Wi-Fi network of its own called AeroSense-Setup, with the password shown on its screen.

Join that network from a phone or laptop. A page opens by itself, listing every network the monitor can see with its signal strength and a padlock where a password is needed. Tap the one you want, type its password on the phone's keyboard, and press Connect. Both the phone and the monitor's screen then show whether it worked.

The network is remembered, so the monitor rejoins it by itself after a power cut. Setup closes on its own once connected, or after five minutes if nobody joins; holding either button cancels it. Cloud uploads pause while setup is open and resume when it closes.

# 6. The Display Pages

AeroSense has seven pages. Pressing Button 2 moves through them in this order:

| # | Page name | Purpose |
| --- | --- | --- |
| 1 | Home | The AQI headline plus all key readings at a glance |
| 2 | Air Quality | Particulate readings in detail, with bars and trend symbols |
| 3 | Gas Monitoring | MQ-2 with a gauge, status word and session peak |
| 4 | Sensor Dashboard | All four raw sensor values in a 2×2 grid |
| 5 | System Status | Device health — sensors, memory, uptime, performance |
| 6 | History Graphs | The last two minutes of readings as line graphs |
| 7 | Statistics | Minimum, average and maximum since power-on or last reset |

## Page 1 — Home

The page you will use most. It is designed to be read from across the room.

The large panel at the top is the Air Quality Index:

- A three-digit AQI number, shown large. It is padded with leading zeros, so an AQI of 42 appears as 042.

- The AQI category beside it — for example GOOD or MODERATE.

- A coloured keyline down the left edge of the panel, which changes colour to match the category. The panel is captioned AIR QUALITY INDEX.

- If the particulate sensor is not currently sending data, this reads - - - and NO DATA.

Below the AQI panel are three particulate cards, and a gas readout along the bottom of the screen:

| Card | Shows |
| --- | --- |
| PM1.0 / PM2.5 / PM10 | The three particulate readings, in µg/m³ (top row) |
| GAS MQ-2 | The combustible gas and smoke level, with its status word beside it, across the bottom of the screen |

The gas readout has a ten-segment bar beneath it. More lit segments means a higher reading — a quick visual cue you can read without looking at the number. During warm-up the readout reads “warming” instead of a value, so a meaningless number is never presented as a measurement.

## Page 2 — Air Quality

The particulate readings in detail.

- Category at the top — a plain-language descriptor based on the PM2.5 reading: Excellent, Good, Moderate, Poor or Hazardous.

- PM1.0, PM2.5 and PM10 each on their own row, with the numeric value and a horizontal bar showing how high it is relative to hazardous levels.

- A trend symbol at the end of each row showing which way the reading is moving.

| Symbol | Meaning |
| --- | --- |
| + (red) | Rising — air is getting worse |
| – (green) | Falling — air is getting better |
| = (grey) | Steady — little change |

Note: This page’s Category is a simple descriptor of the PM2.5 number alone. It is not the same thing as the AQI category on the Home screen, which follows the official index scale. The Home screen AQI is the figure to rely on.

## Page 3 — Gas Monitoring

Detail for the gas sensor. The panel at the top shows MQ-2  SMOKE / LPG — combustible gas and smoke — with its numeric value in the largest type on the page, a coloured gauge running green → yellow → red with a white marker at the current level, and a status word:

| Status word | Meaning | What to do |
| --- | --- | --- |
| LOW | Normal background level | Nothing |
| MEDIUM | Noticeably elevated | Note what changed — cooking, a candle, traffic |
| HIGH | Substantially elevated | Ventilate the room; find the source |
| DANGER | Very high | Ventilate immediately and investigate — see section 10 |
| WARMING UP | Sensor still warming up | Wait for the countdown to finish |

Below the panel, Session peak shows the highest reading seen since power-on or the last statistics reset, with the danger threshold shown beside it under ALARM AT, for comparison. This is useful for catching a brief event you were not watching — a burst while cooking, for instance — that has since cleared.

## Page 4 — Sensor Dashboard

All four sensor values together in a 2×2 grid — PM1.0, PM2.5, PM10 and MQ2 — shown in the proportional face with the unit beneath each value, and with no bars or interpretation. Useful for a quick side-by-side check.

A small teal dot in the lower right pulses steadily. This confirms the device is actively reading and refreshing. If it stops pulsing, the device has stopped responding.

## Page 5 — System Status

The health of the device itself.

| Row | Shows |
| --- | --- |
| ESP32 | A tick mark and RUNNING, confirming the main processor is alive |
| PMS5003 | LINKED or NO SIGNAL — the particulate sensor |
| MQ2 | WARMUP 45s during warm-up, then READY |
| Memory | Free memory in kilobytes |
| Uptime | Time since power-on, as hours:minutes:seconds |
| FPS | Screen refresh rate; the row is labelled FRAMERATE |
| Frame | Time taken to draw the last screen update, in microseconds; the row is labelled RENDER |

FRAMERATE and RENDER are diagnostic figures. You can ignore them in normal use.

## Page 6 — History Graphs

Four small line graphs showing the last two minutes of readings, refreshed every two seconds: PM1.0, PM2.5, PM10 and Gas.

Each graph scales itself automatically to fit the data it holds, so the shape shows you the trend rather than an absolute level. This makes it easy to see the effect of something — opening a window, lighting a stove — within moments.

## Page 7 — Statistics

The minimum, average and maximum of each reading since power-on, updated once per second.

| Row | Reading tracked |
| --- | --- |
| PM2.5 | Fine particulate matter |
| PM10 | Coarse particulate matter |
| MQ2 | Combustible gas / smoke |

Columns are Min, Avg and Max. Dashes mean no data has been collected yet. To clear these figures and start over, use Reset Statistics in the Quick Menu.

Note: Statistics begin accumulating from power-on, which includes the gas sensor warm-up period. The MQ2 Max figure will therefore show the high warm-up values. Use Reset Statistics once warm-up has finished for a meaningful gas figure.

# 7. Understanding the AQI

## What AQI means

AQI stands for Air Quality Index. It converts a pollutant measurement into a single number on a 0–500 scale, where higher always means worse. It exists so you do not need to know what counts as a high or low microgram reading — the index does that interpretation for you.

AeroSense uses the internationally recognised US EPA index, the same scale used by public air quality reports and weather apps, so the number you see here is directly comparable with those.

## How AeroSense calculates it

AeroSense calculates the AQI from its particulate readings, using the official breakpoint tables and interpolation method:

- A sub-index is calculated from the PM2.5 reading. PM2.5 is the primary pollutant.

- A second sub-index is calculated from the PM10 reading.

- The higher of the two becomes the AQI shown on screen.

Taking the worse of the two — rather than averaging them — is how the standard index is defined. It ensures that a problem with either pollutant is reported honestly instead of being diluted.

Note: The MQ-2 gas sensor is not included in the AQI. It gives a relative reading, not a calibrated concentration, and there is no scientifically valid way to convert it into index units. It is monitored and alarmed separately, on the Gas Monitoring page.

## Reading the categories

| AQI | Category on screen | What it means | What to do |
| --- | --- | --- | --- |
| 0–50 | GOOD | Air quality is satisfactory. Little or no risk. | Nothing. Enjoy it. |
| 51–100 | MODERATE | Acceptable. Unusually sensitive people may notice mild effects. | No action needed for most people. |
| 101–150 | SENSITIVE | Unhealthy for sensitive groups — children, older adults, and people with asthma or heart conditions. | Sensitive people should limit prolonged exertion indoors. Ventilate or filter. |
| 151–200 | UNHEALTHY | Everyone may begin to feel effects. | Reduce activity. Ventilate, close windows to outdoor sources, or run an air purifier. |
| 201–300 | V.UNHEALTHY | Very unhealthy. Health warnings apply to everyone. | Avoid exertion. Improve air quality actively. Consider leaving the area. |
| 301–500 | HAZARDOUS | Emergency conditions. The entire population is affected. | Leave the area if possible and address the source. |

The category and its colour update automatically as the reading changes — there is nothing to set.

## Why AQI sometimes shows “- - -”

The AQI panel shows - - - and NO DATA whenever the particulate sensor is not currently delivering valid readings. AeroSense deliberately shows nothing rather than guessing. This is normal for the first few seconds after power-on. If it persists, see section 9.

# 8. Warnings and What to Do

AeroSense watches for two specific conditions and interrupts whatever page you are on with a full-width warning banner.

| Warning | Triggered by | What to do |
| --- | --- | --- |
| Smoke Detected! | MQ-2 reaching a dangerous level — smoke or combustible gas | Ventilate immediately. Check for a gas leak, unattended cooking, or burning. Leave the room if the cause is not obvious. |
| Poor Air Quality | Overall air quality assessment falling to a poor or hazardous level | Ventilate or filter the air. Check for a particulate source such as cooking, smoke or dust. |

## How the banner behaves

- The banner stays on screen for about 3 seconds, then clears by itself and returns you to the page you were on.

- The same warning will not re-appear for about 6 seconds afterwards, so a borderline reading does not flood the screen.

- If the condition persists, the warning will keep returning. A repeating warning means the condition is still present.

- Warnings clear themselves — there is nothing to acknowledge or dismiss.

## Important limitations

Gas warnings are switched off during the first 60 seconds after power-on while the sensor warms up. AeroSense cannot alert you to gas during that first minute.

AeroSense does not detect carbon monoxide. It has no CO sensor. Carbon monoxide is colourless and odourless, and nothing on this device will warn you about it. Fit a separate approved CO alarm.

Read the safety notes in section 10 before relying on these warnings.

# 9. Troubleshooting

| Symptom | Likely cause | What to do |
| --- | --- | --- |
| Screen stays blank | No power | Check the USB cable and power source. Try a different cable — charge-only cables are a common culprit. |
| AQI shows - - - / NO DATA for more than a minute | The particulate sensor is not sending readings | Check the System Status page. If PMS5003 shows No Signal, check the sensor is connected and that its small fan is running. Power off and on. |
| Gas reading very high just after switch-on | Normal warm-up behaviour | Nothing — wait for the 60-second countdown to finish. See section 4. |
| Gas reading still high well after warm-up | Real gas present, or the sensor needs longer to settle | Ventilate the room and watch the reading. If it falls, there was a source. If it stays high, leave the device powered on for a few hours — these sensors stabilise further with extended use. |
| WARMING UP never changes to a status word | Warm-up has not completed | Warm-up is 60 seconds from power-on. If it persists far beyond that, power off and on. |
| Screen frozen; teal dot on the Dashboard not pulsing | Device has stopped responding | Disconnect power, wait five seconds, reconnect. |
| Statistics show a huge Max value for MQ2 | Warm-up readings were recorded | Use Reset Statistics in the Quick Menu after warm-up finishes. |
| Buttons feel unresponsive | A warning banner is showing | While a warning banner is on screen the page is held still — wait about 3 seconds. |

Connection indicator is not green. AeroSense uses 2.4 GHz Wi-Fi and has no 5 GHz radio, which is by far the most common cause — check the network band first. Then check that the signal reaches where the device sits, and that the stored Wi-Fi details are still current. Sync trouble never affects the readings on screen: the device keeps measuring and displaying normally whatever the network is doing.

# 10. Operating and Safety Notes

## Placement and care

- Use indoors only, on a flat dry surface, ideally at breathing height.

- Keep the air intake and outlet clear. Do not cover the device or place it against a wall.

- Keep away from water, steam, direct sunlight and heat sources.

- Avoid placing it directly in a draught, next to a window, or immediately beside a stove — readings will reflect that spot rather than the room.

- Clean the outside with a dry cloth only. Never use liquids or spray cleaners.

- Power the device from a standard USB supply. Do not open the enclosure.

- For best gas sensor stability, leave AeroSense powered on continuously.

## Data and privacy

If cloud sync is enabled, air quality readings from this device leave your network and are stored in a Google Firebase database belonging to whoever set the device up. The readings are of the air in the room the device sits in, so consider whether that is appropriate before placing it somewhere sensitive. To stop it entirely, have the Wi-Fi details removed from the firmware — the radio is then never switched on.

## Important safety limitations

Please read this section carefully.

- AeroSense is an indicative monitoring instrument, not a certified safety device. It is not a substitute for a certified smoke alarm, carbon monoxide alarm, or gas detector, and must not be used as one.

- Do not rely on AeroSense to protect life or property. Fit approved smoke and carbon monoxide alarms in your home as required by local regulations, and keep them maintained.

- AeroSense does not measure carbon monoxide. There is no CO sensor in this device. Nothing on this screen will warn you about carbon monoxide, which is colourless, odourless and can be fatal.

- The gas sensor is uncalibrated. MQ-2 readings are relative indications of change, not measured concentrations, and the sensor responds to more than one gas.

- No alerting during warm-up. Gas warnings are suppressed for the first 60 seconds after power-on.

- AeroSense is not a medical device. Do not use its readings to make medical decisions. Consult a healthcare professional about health concerns.

- If you suspect a gas leak or carbon monoxide, act on that suspicion — not on this device. Leave the area, ventilate, and contact your gas supplier or emergency services.

# 11. Quick Reference

## Buttons

| Where you are | Button 1 press | Button 2 press | Hold Button 1 | Hold Button 2 |
| --- | --- | --- | --- | --- |
| Any page | Previous page | Next page | Go to Home | Open Quick Menu |
| Quick Menu | Highlight up | Highlight down | Cancel / close | Confirm selection |

## Pages, in order

| # | Page | Key content |
| --- | --- | --- |
| 1 | Home | Large AQI + category; PM1.0 / PM2.5 / PM10; gas readout along the bottom |
| 2 | Air Quality | PM values with bars and trend symbols (+ – =) |
| 3 | Gas Monitoring | MQ-2 gauge; LOW / MEDIUM / HIGH / DANGER; session peak |
| 4 | Sensor Dashboard | All four sensor values; pulsing teal dot |
| 5 | System Status | Sensor health, memory, uptime, frame rate |
| 6 | History Graphs | Two minutes of PM1.0, PM2.5, PM10 and Gas |
| 7 | Statistics | Min / Avg / Max for PM2.5, PM10 and MQ2 |

## AQI at a glance

| 0–50 | 51–100 | 101–150 | 151–200 | 201–300 | 301–500 |
| --- | --- | --- | --- | --- | --- |
| GOOD | MODERATE | SENSITIVE | UNHEALTHY | V.UNHEALTHY | HAZARDOUS |

## Warm-up at a glance

| Question | Answer |
| --- | --- |
| How long? | 60 seconds from power-on, once only |
| What still works? | Everything except gas warnings — AQI, all pages, buttons, graphs |
| What is suppressed? | Gas warnings, and the gas contribution to the air quality assessment |

AeroSense — User Guide · Keep for reference